006156----- SPE-S5060型三向精密微调吸气丝印台

006156-----SPE-S5060 Three -Directions Micro Adjust Vacuum Screen Printing Table

吸气台的工作原理是通过吸气泵的电机带动叶轮旋转，从而对固定容积的泵腔内的空气进行压缩、拉伸形成真空（负压），在泵抽气口处与外界大气压产生压力差，在压力差的作用下，将气体压（吸）入泵腔，再从排气口排出。

吸气丝印机的进气口是通过印刷台面上的几百至几千个小孔，承印物放置在吸气台面上受气体负压形成压力差，从而将承印物吸附在台面上。

吸气丝印机的主要优点：

- 可将承印物紧紧吸附在台面上，防止承印物受外力影响移动；

- 在印刷完成后可防止因为油墨的粘性将承印物带起；

- 准确定位承印物位置后，可提供启动和关闭吸气泵放置承印物更加准确。

The working principle of the vacuum table is to drive the impeller rotation by the motor of the suction pump, so as to compress and stretch the air in the pump chamber with fixed volume to form a vacuum (negative pressure). The pressure difference between the pump air inlet and the external atmospheric pressure is generated. Under the action of the pressure difference, the gas pressure (suction) is put into the pump chamber and then discharged from the exhaust port.

The air inlet of the vacuum screen printing machine is through hundreds to thousands of small holes on the printing surface.The substrate is placed on the suction surface by the negative pressure of the gas to form a pressure difference, so that the substrate adsorption on the surface.

Main advantages of vacuum screen printing machine:

1, the substrate can be tightly adsorbed on the table surface, to prevent the substrate affected by external force to move;

2, After the completion of printing to prevent the substrate move along with the ink because of the viscosity of the substrate;

3, After positioning of the substrate position, it can provide start and close the suction pump placement to place substrate more accurate.

一，产品用途：Application:

适用于各种高精密、精细平面丝网印刷，特别是较轻，较薄的产品，如纸张，塑料片，光盘,信用卡,PCB柔性线路板.以及各种膜、卡、板、纸张。

如各类转印花纸，各类PVC、PET、PC等软质、硬质胶片，各类板、薄膜、纸张、铭牌、等产品。

也可以用于较重较厚的产品精细印刷，如纸盒，玻璃，木板，金属板，标牌，各种塑料板等材料。

不适合用于布片，丝带，绶带，手帕等纺织品类产品。

印刷纺织品类产品建议使用可涂刮台版胶的丝印台。

Suitable for all kinds of high precision and fine plane screen printing, especially for light and thin products, such as paper, plastic sheet, CD, credit card,PCB, as well as all kinds of film, card, board and paper.

Such as all kinds of transfer printing paper, all kinds of PVC, PET, PC and other soft, hard film, all kinds of board, film, paper, nameplate, and other products.

It can also be used for the fine printing of heavy and thick products, such as paper boxes, glass, wood boards, metal plates, signage, various plastic boards and other materials.

Not suitable for cloth, ribbons, handkerchiefs and other textile products.

Printing textile products are recommended to use the scraping pallet glue of the screen.

二，基本功能：Basic functions:

1，本丝印台面使用2毫米不锈钢板激光切割，台面面积500*600毫米；吸气面积526*420毫米；吸气孔直径1毫米；吸气孔间距15毫米；吸气孔数量1044个（横29排，竖36排）。

This screen printing table uses 2 mm stainless steel plate laser cutting, table area is 500*600 mm; Vacuum area is 526*420 mm; Suction hole diameter is 1 mm; The spacing of air holes is 15 mm. The quantity of air holes are 1044 (29 rows in horizontal direction and 36 rows in vertical direction).

2，采用精细三向微调型丝印机头；可通过旋钮进行上下，前后20毫米，左右20毫米微调；

将承印物放置在台面上并开启吸气泵，将网板图案基本与承印物对齐后夹紧网框。

高度调整：将网框平放在台面上，根据承印物高度或印刷要求，转动上下调整把手达到合适的高度，通过后部两个定位把手将高度锁定。

左右微调：将机头两边把手松开，根据需要旋转把手左右微调移动；对版完成后，

将两个把手全部拧紧。

前后微调：首先将夹头上方的四个红色旋钮轻微松开，使用夹头前部两边的微调旋钮松开并根据需要进行前后微调，微调完成后将旋钮拧紧，同时将夹头上方的四个红色旋钮拧紧，

Fine three - directions micro adjust screen printing head;It can be micro adjusted through the knob up and down , forward and back  20 mm, left and right 20 mm ;

Place the substrate on the table surface and start the vacuum pump. Clamp the screen frame after aligning the screen pattern with the substrate.

Height adjustment: place the screen frame flat on the platform. According to the substrate height or printing requirements, rotate the up and down adjustment handle to reach the appropriate height, and lock the height through the two rear positioning handles.

Left and right adjustment: loosen the handle on both sides of the nose, and rotate the handle to move slightly left and right as required; When it is done,tighten both handles.

Back and forward adjustment: first, slightly loosen the four red knobs on the front of the chuck, then loosen the fine-tuning knobs on both sides of the front of the chuck and make front and rear fine-tuning as required. After the fine-tuning, tighten the knob and tighten the four red knobs on the top of the chuck.

- 3,网框夹头宽度393毫米，高度80毫米，可适合50毫米以下厚度的各种网框使用,网框夹头安装有四个网框固定把手，并配置专用网框压条，固定网框受力均匀，网框固定更牢

Screen frame chuck width is 393 mm, height is 80 mm, and it can be suitable for the use of a variety of screen frame with the thickness below 50 mm , screen frame chuck installed with four fixed handle , and the dedicated screen frame depression bar, fixed frame stress uniform, fixed frame more firmly.

- 4，网框抬起静止平衡通过重力平衡盘调整（本机配有4个重力平衡盘，可根据平衡需要单个或4个安装）。重力平衡盘可根据需要进行前后100毫米移动；如网框重量过大，可在平衡架顶部圆孔附加重力物。

- 网框抬起角度通过平衡架下部的螺丝进行调整，可根据需要调整网框抬起后高度。

The static balance is adjusted by gravity balance plate when the screen frame is lifted (the machine is equipped with 4 gravity balance plates, which can be installed individually or 4 installed together according to the need of balance). The gravity balance plate can move forward and backward by 100mm as required; If the frame weight is too big,heavy object can be added on the top of the balance frame gravity hole.

The lifting angle of the screen frame can be adjusted by screws in the lower part of the balance frame. The height of the screen frame after lifting can be adjusted as required.

- 5, 承印物面积较小，吸附力会降低，可将承印物周围的吸气孔用塑料片或纸片等盖住即可加大承印物的吸附力。

When substrate area is small, adsorption will reduce, users can cover the blowholes around the substrate with plastic sheets or paper to increase the adsorption of substrate.

- 6，本机主体采用高级静电喷涂金属表面处理，耐酸碱，耐腐蚀，耐清洁，不生锈，可长期使用；部分部件采用不锈钢材料。

- The main body of the machine adopts advanced electrostatic spraying metal surface treatment, acid and alkali resistance, corrosion resistance, cleaning resistance, no rust, long-term use; Some parts are made of stainless steel.

- 7，配有全套安装工具和详细安装说明书；用户不需另配工具即可轻松安装。

- Equipped with a full set of installation tools and detailed installation instructions; Easy installation without additional tools.

- 出口欧美标准免熏蒸木箱包装，箱内各种金属件均单独包装或隔离摆放，保证各种金属部件不直接接触；

- Export to Europe and America standard fumigation-free wooden box packaging, all kinds of metal parts in the box are individually packaged or placed in isolation to ensure that all kinds of metal parts do not contact directly;

- 本机为桌面式，可另配架子组合成落地式。

- 桌面式整机尺寸：长750毫米；宽500毫米，高400毫米；

- 台面距桌面高度：150毫米；

- 落地式整机尺寸：长750毫米；宽500毫米，高度970毫米；

台面距桌面高度：730-880毫米（配有调整高度支脚，可根据需要调整合适高度）。

- the machine is desktop type, can be combined with another rack into floor type.

- Desktop machine size: 750 mm long; 500 mm wide and 400 mm high;

- Height from the table top: 150 mm;

- Floor machine size: 750 mm long; 500 mm wide and 970 mm high;

- Height between table and pallet top: 730-880mm (with adjustable height foot, adjustable height as needed).

三，适用网框：Applicable screen frame:

本机适合内径尺寸宽500毫米，长600毫米，厚度50毫米内的各种材质网框；

建议印刷面积宽400毫米，长500毫米以内。

The machine is suitable for screen frame of various materials with inner diameter of 500mm wide, length of 600mm and thickness of 50mm.

The recommended printing area is 400 mm wide and 500 mm long.