006293 SPE-1600W型四平台落地式定位旋转式烘干机

006293 SPE-1600W four platform positioning rotary dryer

一，本机为两功能烘干机；

The dryer has two functions :

功能一，可对印刷完成后的产品进行彻底烘干；

Function one : Thoroughly dry the product that finished printing

功能二，可与旋转丝印机配套在印刷过程中对产品进行闪烘（需拆下一个或两个烘干平台），提高印刷效率。

Function two: it can support With rotary screen printing machine in the process of printing on products is conducted by flash drying (need to remove one or two drying platforms), improve the efficiency of the printing.

二，本机功能与特点：

The functions and features of this machine:

1，加热管采用1600W不锈钢螺旋干烧管，热效率高，正常使用1万小时以上。

The heating tubes adopts 1600w stainless steel heating tubes ,high heating efficiency,it can normally be used for about 10000 hours

2，烘干箱加热尺寸40*40厘米；烘干平台尺寸40*60厘米

The size of drying box is 16*16in; the size of platform is 16*24in.

3，四个烘干台位可任意360度旋转；旋转盘下配有可调整高度磁铁吸附台位定位，避免随意转动。

Four platforms can rotate in 360 degree . The rotating disk is equipped with adjustable height magnet adsorption platform positioning to avoid arbitrary rotation.

- 烘干箱可360度旋转并可在任意位置固定。

- The drying box can rotate in 360 degree and can be fixed at any position .

5，根据烘干温度需要提高或降低烘干箱进行调整，烘干机箱可上下移动5厘米；

The temperature can be achieved by adjusting the height of the drying box. The drying box can move 2in from top to bottom.

6，加热管背面的反热铝板使热效率更高；

The back of the heating pipe is hot aluminum plate, its thermal efficiency is higher.

7，底部采用四条支架支撑，整机更稳定；配有脚轮可任意移动。

The bottom adopts four holders,and the machine is more stable;It is equipped with casters , it can move arbitrary.

- 8，底座安装可调整20厘米高度支撑脚8个；配有脚轮；需要移动机器时，将支撑脚松开，支撑碗底部高于脚轮即可移动机器。

- The base is equipped with 8 pieces adjusted 20cm high support feet; Equipped with casters; When it is necessary to move the machine, loosen the supporting foot and when the bottom of the supporting bowl is higher than the casters to move the machine.

9，配置全套安装工具，测试温度表和安装说明书。

Prepare for the full set installation tools, test thermometer and installation instructions.

三，使用方法：

How to use:

1，将烘干机加热箱移动至某个平台上方并开启电源加热；

Move the dryer heating box to the top of the drying platform, turn on the power, start the heating.

2，将温度表放置在该平台的中间开始测温；

Place the thermometer probe in the middle of the platform to start temperature measurement;

3,  15分钟后观察温度表显示数字是否为需要的温度；如温度过高；适当提高烘干箱高度；如温度过低，适当降低烘干箱与平台距离，以达到合适的温度即可使用。

Observe the temperature showed in the thermometer after 15 minutes ;

If the temperature is too high; Adjust the height of the drying box ,if the temperature is too low ,shorten the height of the drying box with the platform to achieve the proper temperature .

辅助功能：与旋转烘干机配套进行印刷过程中的表面烘干（照片）

Auxiliary function: Support with rotary dryer in the process of printing surface drying (photos)

测试参考数据：

室温24C（75F）；工作环境内无流通风；

烘干箱底部距离烘干台面3厘米处温度为180C（374F）

烘干箱底部距离烘干台面5厘米处温度为170C（356F）

烘干箱底部距离烘干台面7厘米处温度为160C（320F）

Test reference data:( following data is just for reference )

Use condition:

Room temperature 24 C(75 F); Work environment without circulation wind;

Bottom of the drying box is 3 cm distant from platform , the temperature is 180c (374 F)

Bottom of the drying box is 5 cm distant from platform , the temperature is 170 c (356F)

Bottom of the drying box is 7 cm distant from platform , the temperature is 160 c (320F)

| 产品编码 | 006290 (110V) 006291 (220V) | Product Code | 006290 (110V) 006291 (220V) |
| --- | --- | --- | --- |
| 产品名称 | SPE4040-1600W型四烘干平台落地旋转式烘干机 | Product Name | SPE4040-1600W four platform floor type rotary dryer |
| 海关编码 | 8419399090 | HS code | 8419399090 |
| 产品颜色 | 黑色 | Product Color | Black |
| 烘干箱尺寸 | 40*40cm | Size of Drying Box | 16*16inch |
| 加热管材料 | 不锈钢螺旋干烧管 | Heating tube material | Stainless Steel Heating Tubes |
| 电压 | 220V/110V | Voltage | 220v/110v |
| 功率 | 1600W | Power | 1600w |
| 插头 | 美标，国标，欧标（可选择） | Plug | American standard,National standard,European standard |
| 机器使用材料 | 钢板 | Machine using material | steel plate |
| 机器表面处理 | 静电喷涂 | Machine surface treatment | Electrostatic spraying |
| 附件材料 | 不锈钢或镀锌，镀镍 | Attachment material | Stainless steel or zinc plating, nickel plating |
| 净重 | 31千克 | Net weight | 68lb |
| 毛重 | 33千克 | Gross weight | 73lb |
| 包装尺寸 | 81*56*28cm | Packing size | 32*22*11in |
| 包装体积 | 0.13立方米 | Packing volume | 0.13 m³ |
| 包装材料 | 出口纸箱 | Packing material | Export fumigation |