本机主要用于精确六色T恤套色印刷，配置六个网框夹头与六个T恤台版工位；

It is mainly used for the six color T-shirt overprinting,it is configured with

six screen frame chucks and six t-shirt pallet stations;

1，台版部分与网框部分均可独立360度旋转；操作者可在任意工位上操作；

Both the pallet part and the screen frame part can rotate independently

at 360 degree;Users can operate at any station;

The chuck of the screen frame can rotate at 360 degree.

2，每个网框臂安装有3个旋钮可对网板微调，网版直接在夹头上加紧，

松开台版臂上部两个固定把手，通过三个微调旋钮对网框进行前，后，左，右15毫米微调，

网版套色对标完成后，将台版臂上部两个固定把手拧紧即可。

Each screen frame arm is installed with 3 knobs to fine-tune the screen plate, screen plate

directly fixed on the clamp, loosen the upper two fixed handle on thepallet arm ,

adjusting the screen frame back and forward, left and right within 15 mm by the three

fine-tuning knobs. When the screen plate color alignment is finished,

the upper two fixed handle on the pallet arm can be tightened.

3，操作者可在任意一个台版工位进行所有网板的套色对标，所有网版对标任意一个台版准确

后，其它台版均达到对标准确，即可准确套色印刷。无需每个台版对标套色。

Users can finish all the screen frame’s overprinting positioning at any pallet station,

All the screen frames’position is correct to any one pallet , the other one pallet will be correct ,

then the user can start overprinting.There is no need to correct the position of every pallet.

4，本机采用可调节高度架子；可以根据操作者身高和工作习惯调整架子高度；

调整范围为台版距离地面58-109厘米；

操作者可根据架子高度站立工作，也可坐在椅子上工作。

The machine adopts adjustable height rack; The height of the rack can be adjusted

according to the operator's height and working habits; The adjustment range

from the ground is 58-109 cm ;The operator can work by standing or sitting on

a chair according to the height of therack.

5，每个网框固定装置均配有精确定位卡轮，

网框臂上的定位卡铁放置于定位卡轮后紧密贴合，保证印刷精确不偏离。

Each screen frame’s fixing device is equipped wth precise positioning wheel,when the

positioning iron on the arm placed in positioning wheel closely,

it can ensure the printing precision .

6，每个网框固定装置均配有卡位大轮，网框臂可在两个卡位大轮之间触碰卡位

大轮后即可轻松落入金属定位卡轮中，还可防止定位卡铁与金属定位卡轮直接发生碰撞。

Each screen frame’s fixing device is equipped wth big lock wheel, screen frame arm

can easily fall into the metal positioning wheel when touch the two big lock

wheels ,and it also can prevent the direct collision between the positioning

iron and the metal positioning wheel .

7，根据网框重量可通过花篮螺丝调整弹簧张力，以达到网框起落平衡；

According to the weight of the screen frame, spring

tension can be adjusted by the basket screws to achieve balance;

8，网框夹头宽度25（10in）厘米；

适合网框外径60（24in）厘米以内，网框厚度4厘米（1.6in）内的各种网框；

Screen frame chuck’s width is (10in) 25 cm ; It is suitable for screen frame

with outside size within (24 in) 60 cm and screen frame with

thickness size is within 4 cm (1.6 in) ;

9，网框夹头可根据承印物高度向上调整5（2in）厘米；

Screen frame chuck can be adjusted up to 5 cm (2 in)according

to the height of substrates;

10，网框夹头旋钮与专用铝合金压板连固定网框，使压板与网框紧密贴合更加稳定；

Screen frame chuck knob fix the screen frame together with the special

aluminum alloy plate, and it can be fit closely and be more stable;

11，本机配置的T恤型台版长55厘米（22in），45厘米（18in），厚度1.8厘米（0.7in）

采用表面三聚氰胺高密度家具板，表面为细微麻面，更适合印刷T恤时涂刮定位台版胶。

T-shirt type pallet of the machine is 55 cm (22 in) long, 45 cm(18in) wide,

the thickness is 1.8 cm (0.7 in); Using furniture surface melamine high

density board, the surface with small pits, and it is more suitable for printing

T-shirt when scraping the pallet glue.

12,本架子可以根据身高需要调节五个高度范围;

This shelf can adjust five height ranges according to height requirements;

- 本架子调节高度最低的时候直径95厘米，调节高度最高的时候直径135厘米;

The diameter of the lowest acljusting height of this shelf is 95 cm,and the diameter of the highest adjusting height is 135 cm;

- 本架子要移动的时候可以抬高四个支撑脚，让轮子着地，就可以推着移动.

When this shelf wants to move, it can raise four supporting feet,so that the wheel lands, it can be pushed to move.