006560 A250型桌面固定无微调丝印夹头

A250 Desktop Fixed Screen Printing Clamp

一，基本功能：Basic functions:

1，夹头宽度250毫米，高度80毫米，可适合60毫米以下厚度的各种网框使用,网框夹头安装有三个网框固定把手，网框固定更牢固。

The chuck is 250mm wide and 80mm high, and it is suitable for all kinds of screen frames with thickness below 60mm.There are three grille fixing handles installed in the chuck , it is stronger to fix the screen frame.

- 本丝印夹头需通过自攻丝直接固定在木质工作台上或M8螺栓穿透金属或木质工作台固定使用（本机配置有木螺丝4个和M8的螺丝螺帽4套。

The screen chuck shall be directly fixed on the wooden working table by self-tapping screws or fixed on the metal or wooden working table by self-tapping M8 bolts (the machine is equipped with 4 screws and 4 sets of M8 screws and nuts).

- 网框抬起重力平衡盘（本机配有2个重力平衡盘，可根据平衡需要单个或2个安装）可根据需要进行前后100毫米移动；如网框重量过大，可在平衡架顶部圆孔附加重力物。平衡架中部安装有可调整网框抬起高度把手，可根据需要调整网框抬起后高度。

- Lifting gravity balance plate of the screen frame can move 100mm forward and back according to needs (the machine is equipped with 2 gravity balance plates, which can be installed individually or together according to the need of balance). If the frame weight is too big, heavy object can be added on the top of the balance frame gravity hole. The middle part of the balance frame is equipped with a height handle which can adjust the height of the screen frame after lifting according to the need.

- 通过松紧夹头合页螺丝可将夹头升降40毫米，适合厚度40毫米内的各种承印物印刷。

- The chuck can be raised or lowered 40 mm by loosening the hinge screw of the elastic chuck, and it is suitable for printing of various substrates within the thickness of 40 mm.

- 配有长宽500*330毫米，厚度2毫米平整高级橡胶板一张，直接放置在桌面上，可作为印刷承印物底板使用。

- Equipped with 500* 330mm in length and width and 2mm in thickness of a flat high-grade rubber board, which can be directly placed on the desktop and used as the substrate for printing.

- 本机主体采用高级静电喷涂金属表面处理，耐酸碱，耐腐蚀，耐清洁，不生锈，可长期使用；部分部件采用不锈钢材料。

- The main body of the machine adopts advanced electrostatic spraying metal surface treatment, acid and alkali resistance, corrosion resistance, cleaning resistance, no rust, long-term use; Some parts are made of stainless steel.

- 配有全套安装工具和详细安装说明书；用户不需另配工具即可轻松安装。

- Equipped with a full set of installation tools and detailed installation instructions; Easy installation without additional tools.

8，采用出口欧美标准纸箱包装，箱内各种金属件均单独包装或隔离摆放，保证各种金属部件不直接接触；箱内空档部分全部用泡沫板材或气泡袋填充为满箱，保证运输过程中纸箱不变形，防止箱内机器部件受损。

Adopts the standard carton packaging exported to Europe and America, all kinds of metal parts in the box are individually packaged or placed in isolation, to ensure that all kinds of metal parts do not contact directly; All the empty parts in the box are filled with foam sheets or bubble bags to ensure that the carton does not deform during transportation and prevent damage to the machine parts in the box.

二，产品用途：Application:

适用于各种平面产品印刷，如电子线路板，纸盒，玻璃，木板，金属板，标牌，各种塑料板等有一定重量的材料。

Suitable for all kinds of plane product printing, such as electronic circuit board, paper box, glass, board, metal plate, sign, all kinds of plastic board, etc.

不建议用于布片，丝带，绶带（如印制布片需台面涂刮台版胶），纸张，薄塑料片等较轻的材料（这些材料因为太轻，印刷完毕，网框抬起后可能会与网板背面粘结在一起无法取下造成废品）

纸张，薄塑料片等较轻的材料建议使用吸气丝印台；

布片，丝带，绶带等纺织品印刷建议使用可涂刮台版胶的丝印台。

It is not recommended to use for cloth, ribbons, ribbons (if the printed cloth needs to be coated with scraping board glue), paper, thin plastic sheets and other light materials (these materials are too light, after the printing, the screen frame may be stuck together with the back of the screen board and cannot be removed, causing waste products).

Paper, thin plastic sheets and other lighter materials are recommended to use suction screen printer ;

Cloth, ribbon, and other textile printing is recommended to use the scraping pallet glue of the screen.

三，适用网框：Applicable screen frame:

本机适合内径尺寸宽350毫米，长500毫米，厚度60毫米内的各种材质网框；

建议印刷面积宽300毫米，长450毫米以内。

The machine is suitable for screen frame of various materials with inner diameter 350 mm wide, length 500 mm and thickness 60 mm.

The recommended printing area is 300 mm wide and 450 mm long.

照片图示文字：

1, 网板升降高度通过松紧移动合页螺丝可上下调整40毫米

The lifting height of the screen frame can be adjusted up and down 40 mm by loosening and moving elastic and  hinge screws.

- 网板抬起角度通过重力支撑板上下螺丝调整0-45度

- The lifting angle of the screen frame is adjusted 0-45 degrees by the upper and lower screws of the gravity support plate

- 橡胶底板也根据需要摆放，适合印刷需要。

- The rubber base plate is also arranged according to the need, suitable for printing.

3，重力板可根据网板重量前后调整达到合适的平衡，也可增减重力板数量达到合适的平衡。

The gravity plate can be adjusted back and forth according to the weight of the screen plate to reach a proper balance, and the quantity of gravity plates can be increased or decreased to reach a proper balance.

006563 A480型桌面固定无微调丝印夹头

A480 Desktop Fixed Screen Printing Clamp

一，基本功能：Basic functions:

- 夹头宽度480毫米，高度80毫米，可适合60毫米以下厚度的各种网框使用,网框夹头安装有四个网框固定把手，网框固定更牢固。

- The chuck is 480mm wide and 80mm high, and it is suitable for all kinds of screen frames with thickness below 60mm.There are three grille fixing handles installed in the chuck , it is stronger to fix the screen frame.

- 本丝印夹头需通过自攻丝直接固定在木质工作台上或M8螺栓穿透金属或木质工作台固定使用（本机配置有木螺丝12个和M8的螺丝螺帽6套。

The screen chuck shall be directly fixed on the wooden working table by self-tapping screws or fixed on the metal or wooden working table by self-tapping M8 bolts (the machine is equipped with 12 screws and 6 sets of M8 screws and nuts).

- 网框抬起重力平衡盘（本机配有4个重力平衡盘，可根据平衡需要单个或多个安装）可根据需要进行前后100毫米移动；如网框重量过大，可在平衡架顶部圆孔附加重力物。平衡架中部安装有可调整网框抬起高度把手，可根据需要调整网框抬起后高度。

- Lifting gravity balance plate of the screen frame can move 100mm forward and back according to needs (the machine is equipped with 4 gravity balance plates, which can be installed individually or together according to the need of balance). If the frame weight is too big, heavy object can be added on the top of the balance frame gravity hole. The middle part of the balance frame is equipped with a height handle which can adjust the height of the screen frame after lifting according to the need.

- 通过松紧夹头合页螺丝可将夹头升降40毫米，适合厚度40毫米内的各种承印物印刷。

- The chuck can be raised or lowered 40 mm by loosening the hinge screw of the elastic chuck, and it is suitable for printing of various substrates within the thickness of 40 mm.

- 配有长宽500*600毫米，厚度2毫米平整高级橡胶板一张，直接放置在桌面上，可作为印刷承印物底板使用。

- Equipped with 500* 600mm in length and width and 2mm in thickness of a flat high-grade rubber board, which can be directly placed on the desktop and used as the substrate for printing.

- 本机主体采用高级静电喷涂金属表面处理，耐酸碱，耐腐蚀，耐清洁，不生锈，可长期使用；部分部件采用不锈钢材料。

- The main body of the machine adopts advanced electrostatic spraying metal surface treatment, acid and alkali resistance, corrosion resistance, cleaning resistance, no rust, long-term use; Some parts are made of stainless steel.

- 配有全套安装工具和详细安装说明书；用户不需另配工具即可轻松安装。

- Equipped with a full set of installation tools and detailed installation instructions; Easy installation without additional tools.

8，采用出口欧美标准纸箱包装，箱内各种金属件均单独包装或隔离摆放，保证各种金属部件不直接接触；箱内空档部分全部用泡沫板材或气泡袋填充为满箱，保证运输过程中纸箱不变形，防止箱内机器部件受损。

Adopts the standard carton packaging exported to Europe and America, all kinds of metal parts in the box are individually packaged or placed in isolation, to ensure that all kinds of metal parts do not contact directly; All the empty parts in the box are filled with foam sheets or bubble bags to ensure that the carton does not deform during transportation and prevent damage to the machine parts in the box.

二，产品用途：Application:

适用于各种平面产品印刷，如电子线路板，纸盒，玻璃，木板，金属板，标牌，各种塑料板等有一定重量的材料。

Suitable for all kinds of plane product printing, such as electronic circuit board, paper box, glass, board, metal plate, sign, all kinds of plastic board, etc.

不建议用于布片，丝带，绶带（如印制布片需台面涂刮台版胶），纸张，薄塑料片等较轻的材料（这些材料因为太轻，印刷完毕，网框抬起后可能会与网板背面粘结在一起无法取下造成废品）

纸张，薄塑料片等较轻的材料建议使用吸气丝印台；

布片，丝带，绶带等纺织品印刷建议使用可涂刮台版胶的丝印台。

It is not recommended to use for cloth, ribbons, ribbons (if the printed cloth needs to be coated with scraping board glue), paper, thin plastic sheets and other light materials (these materials are too light, after the printing, the screen frame may be stuck together with the back of the screen board and cannot be removed, causing waste products).

Paper, thin plastic sheets and other lighter materials are recommended to use suction screen printer ;

Cloth, ribbon, and other textile printing is recommended to use the scraping pallet glue of the screen.

三，适用网框：Applicable screen frame:

本机适合内径尺寸宽550毫米，长650毫米，厚度60毫米内的各种材质网框；

建议印刷面积宽500毫米，长600毫米以内。

The machine is suitable for screen frame of various materials with inner diameter 550 mm wide, length 650 mm and thickness 60 mm.

The recommended printing area is 500 mm wide and 600 mm long.