006580------SPE-3XWT280型精密微调丝印夹头

006580----- SPE-3XWT280 Precision Micro Adjust Screen Clamp

本丝印夹头机为三向精密微调型；可通过旋钮进行上下170毫米，前后20毫米，左右20毫米微调；

The machine adopts fine three - directions micro adjust screen printing head;It can be micro adjusted through the knob up and down 170mm , forward and back 20 mm, left and right 20 mm ;

1，高度调整：将网框平放在台面上，根据承印物高度或印刷要求，转动上下调整把手达到合适的高度，通过后部两个定位把手将高度锁定。

Height adjustment: place the screen frame flat on the platform. According to the substrate height or printing requirements, rotate the up and down adjustment handle to reach the appropriate height, and lock the height through the two rear positioning handles.

2，左右微调：将机头两边把手松开，根据需要旋转把手左右微调移动；对版完成后，

将两个把手全部拧紧。

Left and right adjustment: loosen the handle on both sides of the nose, and rotate the handle to move slightly left and right as required; When it is done,tighten both handles.

- 前后微调：首先将夹头上方的四个红色旋钮轻微松开，使用夹头前部两边的微调

- 旋钮松开并根据需要进行前后微调，微调完成后将旋钮拧紧，同时将夹头上方的四个红色旋钮拧紧，

Back and forward adjustment: first, slightly loosen the four red knobs on the front of the chuck, then loosen the fine-tuning knobs on both sides of the front of the chuck and make front and rear fine-tuning as required. After the fine-tuning, tighten the knob and tighten the four red knobs on the top of the chuck.

- 4， 网框夹头宽度280毫米，高度80毫米，可适合50毫米以下厚度的各种网框使用,网框夹头安装有三个网框固定把手，并配置专用网框压条，固定网框受力均匀，网框固定更牢固。

- Screen frame chuck width is 280 mm, height is 80 mm, and it can be suitable for the use of a variety of screen frame with the thickness below 50 mm , screen frame chuck installed with three fixed handles , and the dedicated screen frame depression bar, fixed frame stress uniform, fixed frame more firmly.

- 5，网框抬起静止平衡通过重力平衡盘调整（本机配2个重力平衡盘，可根据平衡需要单个或2个安装）。重力平衡盘可根据需要进行前后100毫米移动；如网框重量过大，可在平衡架顶部圆孔附加重力物。

- 网框抬起角度通过平衡架下部的螺丝进行调整，可根据需要调整网框抬起后高度。

The static balance is adjusted by gravity balance plate when the screen frame is lifted (the machine is equipped with 2 gravity balance plates, which can be installed individually or 2 installed together according to the need of balance). The gravity balance plate can move forward and backward 100mm as required; If the frame weight is too big,heavy object can be added on the top of the balance frame gravity hole.

The lifting angle of the screen frame can be adjusted by screws in the lower part of the balance frame. The height of the screen frame after lifting can be adjusted as required.

- 6，本机主体采用高级静电喷涂金属表面处理，耐酸碱，耐腐蚀，耐清洁，不生锈，可长期使用；部分部件采用不锈钢材料。

- The main body of the machine adopts advanced electrostatic spraying metal surface treatment, acid and alkali resistance, corrosion resistance, cleaning resistance, no rust, long-term use; Some parts are made of stainless steel.

- 7，出口欧美标准免熏蒸木箱包装，箱内各种金属件均单独包装或隔离摆放，保证各种金属部件不直接接触；

Export to Europe and America standard fumigation-free wooden box packaging, all kinds of metal parts in the box are individually packaged or placed in isolation, to ensure that all kinds of metal parts do not contact directly;

8，适用网框：Applicable screen frame:

本机适合内径尺寸宽330毫米，长430毫米，厚度50毫米内的各种材质网框；

建议印刷面积宽280毫米，长380毫米以内。

The machine is suitable for screen frame of various materials with inner diameter of 330mm wide, length of 430mm and thickness of 50mm.

The recommended printing area is 280 mm wide and 380 mm long.