Manual for the portable receiver

● Call button registering

1) Standby mode, press key [FUN] 2 seconds, F1 appears on the LED screen.

2) Keep pressing key FUN till 001 appears.

3)  Press key FUN to move the cursor and press key ENT to choose the number wanted.

4)  Press the call button to be registered. If the portable receiver buzzes with the registered number flashing on the screen, it means the registration is OK.

5)  To register all the call buttons needed, please repeat step (3) ~ (4) after the first one finished.

6)  After registration, Keep pressing key FUN till F1 appears.

7) Keep press key ENT till back to standby mode. The registration is done.

※Special notes

1)  10 numbers can be saved and rolling displayed by the portable receiver. For example, when “1 002” appears on the screen, it means there are 2 paging numbers, 002 is the latest one, and 1 is the quantity of the waiting paging number.

2)  For the multi-key call buttons, the last digit on the LED screen is the detailed function, 6 means service, 8 means bill. For example, “001-6” means that 001 is paging for service. “001-8” means that 001 is paging for bill. When the key CANCEL of the call button is pressed, the calling number on the LED screen disappears and it means the service is done.

● Prompting mode setting

1) Standby mode, press key FUN 2 seconds, F1 appears.

2) Press key ENT repeatedly till F2 appears.

3)  Keep pressing key FUN till a number between 001~005 appears. Press key ENT to choose the number wanted (number 000 means beep prompt, while number 001~005 means vibration prompt. The vibration time can be changed from 1 second to 5 seconds as required.)

4) Keep pressing key FUN till F2 appears.

5) Keep pressing key ENT till back to standby mode. The setting is done.

● Display time setting

1) Standby mode, press key FUN 2 seconds, F1 appears.

2) Press key ENT repeatedly till F3 appears.

3) Keep pressing key FUN, a number between 001~099 appears. Press key ENT to choose the display time wanted. The number 001 to 099 is the display time, from 1 second to 99 seconds accordingly. The number 000 means unlimited display.

4) Keep pressing key FUN till F3 appears.

5) Keep pressing key ENT till back to standby mode, the display time setting is done.

● Deleting registered numbers

1) Standby mode, press key FUN 2 seconds, F1 appears.

2) Press key ENT repeatedly till F4 appears on the LED screen.

3)  Keep pressing key FUN, the minimum registered number appears.

4) Press key ENT repeatedly to choose the number to be deleted. Then, Keep pressing key ENT to delete the number.

And the next registered number appears on the screen after the deleting.

5) Keep pressing key FUN till F4 appears.

6) Keep pressing key ENT till back to standby mode. Deleting is done.

● Initializing the portable receiver

1) Press key [FUN] 2 seconds and F1 appears on the screen.

2) Press key [ENT] repeatedly till F5 appears.

3) Keep pressing [FUN] till [000] appears, and press key [ENT] 3 seconds to start the initializing. Then numbers keep changing on the LED screen. When [000] appears again, the initialization is done.

4) Keep pressing key [FUN] till F5 appears

5) Keep pressing key [FUN], and back to standby mode (time mode). The initializing is done.

● Time setting

1) Standby mode, press key FUN 2 seconds, F1 appears.

2) Press key ENT repeatedly till F6 appears on the LED screen.

3) Keep pressing key FUN, the current time, such as 08:10, appears.

4) Press key ENT to regulate the time. Press key FUN to move the cursor.

5) After time setting, Keep pressing key FUN till F6 appears.

6) Keep pressing key ENT, and back to standby mode. Time setting is done.

※ Time returns to zero every time after the battery picked out and needs resetting.

● Number display mode setting (all-digital display or digits beginning with an English letter)

1) Standby mode, press key FUN 2 seconds, F1 appears on the LED screen.

2) Press key ENT repeatedly till F7 appears.

3) Keep pressing key FUN till a number appears. (If the number is between 00 and 17, it’s all-digital display (default setting); if the number is between 18 and 89, it’s digits beginning with an English letter display, the English letters includes A,B,C,D,E,F,H,P,L and J). Press key ENT repeatedly to choose the number wanted. Keep pressing key ENT can help to expedite the number changing.

4) Keep pressing key FUN till F7 appears.

5)  Keep pressing key ENT, and back to standby mode. The setting is done.

● Deleting calling numbers

Press key ENT to delete the calling number implied on the display.

● Sleep mode setting

Keep pressing key ENT 5 seconds on standby mode to activate the sleep mode（Number 1 appears on the left of the LED screen.）. Then, the portable receiver is same as powered off except time display function. Press key ENT 5 seconds again to wake the portable receiver up ( Number 1 on the left of the screen disappears accordingly).