Ice Shaver

1. Operation Instruction

- Check whether the power voltage conforms to that on the designation plate and the power should connect to the reliable ground wire.

- Raise ice crushing handle to the upperstop,connest to the power,hurn on the master switch and place the ice basin on the underframe with the ice cubes put into the ice crushing barrel.(Note:Quantity of the ice cubes should not exceed the ice crushing barrel level).Then press down the ice predding handle to make the protective switch automatically turned on and crush ice.After finishing,please raise the ice crashing handle to the upper stop again and repeat the above mentioned operation process

2.Keep Clean

After using the ice crushing,the switch should be turned off.Disconnect the power plug and wipe dry the body,then place the machine in the dry place.

- Cautions

When using the outlet of ice cubes cannot be jammed or the cutter head may get stuck and the motor be burnt out.If the above mentioned conditions appear,please find out the reasons immediately,cut off the power and raise the handle.

- Technical Paramaters

| Voltage | 110v |
| --- | --- |
| Power | 300W |
| Frequency | 50Hz-60Hz |
| Weight | 5.5-5.7KG |
| Revolution | 2000R/M |
| Crushing weight | 65kg/hour |