6 Color 6 Station Screen Press Installation Manual

One Installing Rack
a. Install four balance foot stands.

b. Install four bottom level beams.

c. Install four vertical beams.

d. Install four upside level beam

Note: Do not tighten all screws before the whole rack is finished. 
(When the mainframe bottom turntable installation has been finished, then you can tighten all the screws of the rack.)

Two Installing Mainframe Bottom Turntable

Put the mainframe bottom turntable on the rack, tighten the four connecting screws and then tighten all screws of the rack.

Note: Tighten all screws when the mainframe bottom turntable installation is finished.

Three Installing Pallet Support Arms and Orientation Blocks

a. Install the pallet support arm.

b. Install the orientation blocks.

c. Install the control pad for balancing printing head support arm during up-and-down adjusting process.

Notes:

1. Do not tighten the screws when the pallet support arm installation is finished.

2. Do not tighten the screws when the orientation blocks installation is finished.
(When the upper turntable installation and the printing head adjustment are finished, then you can tighten the screws.)

Four Installing Mainframe Upper Turntable

Install the upper turntable on to the mainframe.

Note: The end of one hexagon screw on the central bearing of the upper turntable must aim at the central shaft of the bottom turntable; then tightened the two screws.

Five Installing Parts of Mainframe Upper Turntable

a. Install the collet knobs (12 pcs) of the printing head.

b. Install the stainless steel rings (12 pcs).

c. Connect the printing head collets and the position collet fastenings by tightening 12 screws.

d. Fix the printing head collets and fastenings on to the pallet support arms by tightening the 12 screws.

e. Connect the springs and the stainless steel rings of collets by loosened turn-buckles.

f. Fasten the position knobs (6 pcs) for adjusting up-and-down height of the printing head support arm.

g. Adjust the turn-buckle to a proper strength to make it easy for support arm uplifting and down.

Close Attention: All six arms of printing head have been position-adjusted and all the screws have been tightened before delivery; please do not adjust these screws of the printing head support arms during installation and operation.

Six Micro Adjusting the Mainframe for Precision Position

a. Uplift all the printing heads and twirl the bottom turntable; then put them down according to the center of the orientation blocks.

b. Micro adjust the black pallet support arms to make sure it is easy to put up and down the printing head support arms, then tighten the screws of connecting pallet support arms and the bottom turntable.

c. Fasten the orientation blocks on the pallet support arms.

After the adjustment of the black pallet support arms, with all the printing heads putting down, micro adjust the orientation block to make sure it is easy to put up and down the printing head support arms, tighten screws of the orientation blocks, and then they have been fixed on the pallet support arms.

d. Keep the two big wheel blocks in two sides and fasten them to prevent problems of printing deviating.

e. Keep the two bottom orientation blocks inside and tighten them.

f. Uplift all the printing heads, rotate the upper or bottom turntable and then randomly put them down to check the condition of going up and down.

Close Attention: If some printing heads can not be well put down, you can only adjust the pallet support arm, the block wheel stand and the small orientation block; the printing head support arms mustn’t be adjusted.

Seven Installing Pallet

a. Install the bracket on the pallet according to the position of holes.

b. Install knobs on the pallet brackets.

c. Fasten the pallets on the support arms.

Put down the printing heads, adjust and position the pallets on the place of 3-5mm (0.1” - 0.2”) distance from the printing head collets by tighten the bottom bracket knobs and the screws on the two sides of the brackets.

Eight Finished Installation

ASC365 International Ltd
 http://asc365.com