Special instruction of APE8800：

Time adjusting of perpetual calendar:

1) Standby mode, press key [FUN] till digit flashes.

2) Press key [MOVE] to choose position of adjusted digit.

3) Press key [UP] or [DOWN] to choose the digit wanted.

4)  Press key [FUN] back to standby mode. Done.

Instruction of remote control

Function: deleting registered number

1) Choose the No.300 position to finish the registering of remote control with manual operation. Press any key in registering.

2) Instruction of keys: C key——deleting the display number on the screen.

D key——deleting all of the numbers on the screen.

Call button registering:

1) Standby mode, press key [FUN] 3 seconds till[F1] appears on the screen.

2) Press key [ENT], [001---] appears(the first position),press key [UP], [DOWN] to choose other poison.(300 positions in all, only one register number in one position. Next number should be registered in position [002---] after the registration in position [001---] and so on. ----the step of position choice.

3) Press key [ENT] again, and [001-01] appears. Press key[ UP], [[MOVE]] or [DOWN] to choose the register number wanted. ----the step of register number.

4) Press the call button to be registered, the register number flashes on the screen with music prompt and then stops.

5)  Press key [ESC], and [F1] appears.

6)  Press key [ESC] again, back to standby mode. The registration is finished.

7) Repeat step 2）~4）to register all the call buttons you need, and then press key [ESC] twice back to standby mode. Done.