WF—007A

Wire filling machine

（Argon arc welding wire feeding machine）

An instruction manual

Directory

Safety rules

Important statement

The first chapter    Product introduction

- attention

- Introduction

- Use

- Function setting and specifications

- Cold filler wire machine standard parts list

- The main function characteristic

- The main technical parameters

- Component model specification and selection table

The second chapter   Installation guide

- Connection and installation of each component of the filling wire machine

- Wire installation

- Connection and installation machine with cold filler wire argon arc welding machine

- The selection of supporting parts

- Installation notes

The third chapter    Operate Guidebook

- Confirmation of events before the operation

- Manual argon arc welding wire filling machine operation steps in the process of

- Automatic argon arc welding machine welding process of wire filling machine operation steps

- The choice of matching of welding torch

- The selection of supporting power

- When in use, the welding torch operation points

- When in use, the matters needing attention

The fourth chapter  Control panel function parameter adjusting guide

- Wire feeding speed selection

- Delay wire feeding time selection

- Wire feeding back to the time selection

- A pulsed wire feeding mode or continuous wire feeding mode selected

- A pulsed wire feeding mode operation work cycle wire feeding time selection

- A pulsed wire feeding mode operation work cycle feeding pause time selection

The fifth chapter  Fault diagnosis and maintenance

- Fault diagnosis and treatment table

- Daily maintenance

The sixth chapter  Figure

- Wire feeding machine front view

- TIG welding with wire feeding head installation diagram

- Side view of wire feeding machine

- Side view of wire feeding machine

- Argon arc welding wire feeding machine system schematic diagram of connection

Safety rules:

Warning: procedures and unsafe operation will lead to personal injury or death!

All the equipment of the user, the user and auxiliary personnel, must read and understand the safety rules.

- To prevent electric shock:

Touch the live parts will result in serious injury and fatal blow.

(1) Do not touch the live parts.

(2) don't damp environment work;

(3) put on dry insulating gloves and protective clothing for work.

(4) in the installation and maintenance of the equipment before cut off all power supply.

(5)turn off the power when not in use.

(6) according to the user's manual welding power supply grounding installation guide;

(7) do not use the old cable joints and too small and poor contact;

(8) don't put the cable wound on the body;

(9) do not contact at the same time the torch electrode and the grounded objects and loop.

(10) the use of good maintenance of equipment, timely repair and replacement of equipment is bad.

2. Provide protection against smoke and gas harm:

Inhalation of smoke and harmful gases will harm the human body health.

(1) make the head away from the smoke

(2) the use of proper ventilation to smoke and harmful gas go out from work and the breathing zone.

(3) if the ventilation system of supplementary fit, use of respiratory protection device.

(4) through reading all the material safety data sheet or the manufacturer's instructions.

3.Good protection of compressed gas cylinders:

Will usually use compressed gas cylinders when welding, be careful with.

(1) avoid the compressed gas cylinders from overheating, mechanical shock and arc damage.

(2) compressed gas cylinders tied on a bracket, a wall or platform. The installation is firm.

(3) compressed gas cylinders should be far away from the welding or other electric circuit.

(4) do not make the welding electrode contact compressed gas cylinders.

4. Prevent arc damage eyes and skin, prevent noise damage hearing

The welding arc to produce a strong radiation of heat and ultraviolet radiation burns to the eyes and skin. The working process of the noise can damage hearing.

(1)with appropriate filter is arranged on the helmet;

(2) the protective screen or baffle to protect other personnel;

(3) wear protective clothing and protective shoes.

5. to prevent fire and burn

Heat the work pieces, thermal equipment, splash and spark may cause fire and burns.

(1) in the work area wear appropriate eye protection products, face and body.

(2) before the operation to make the work piece and the equipment fully cooling.

(3) not in the welding of flammable materials near

(4) vigilant catch fire, the fire extinguishers placed next to.

- 1.Special protection case

(1) containers and materials not cutting fitted or contact with dangerous goods;unless it has been properly cleaned and checked.

(2) don't Hange painted and work piece electroplating, unless there is a clear poisonous smoke and gas efficient equipment.

(3) due to the welding affect pacemaker, all cardiac pacemaker users should be far away from the welding work area, consult a doctor should be close to the front.

7.Provide adequate maintenance for equipment

Equipment maintenance and inappropriate may make the effect becomes worse,even the most serious consequence is caused by fire and electric shock,resulting in personal injury and death.

(1) by qualified personnel to install, fault detection and maintenance. Avoid unlicensed live working.

(2) part of the power supply must be pre cut off power supply maintenance

(3) in accordance with the rules for safe operation and maintenance of cable, wire connecting line, power supply line and the power supply socket. Do not operate in doubt state.

(4) don't mess with any equipment and accessories, make the equipment from overheating, moist, greasy, corrosive gases and weather environment.

(5) correctly placed all safety facilities and tools.

(6) special plane, shall not change the purpose of use.

*** Important statement

The specification of the right of final explanation and modification, the company will not regularly on this instruction manual modified without prior notice.

The first chapter    Product introduction

- 1.Introduction

WF-007A Cold filler wire machine (argon arc welding wire feeding machine) is used for argon arc welding, laser welding, plasma welding, brazing welding process, without preheating wire filling equipment for the molten pool.

- 2.Operation:

(1)as the manual argon arc welding process with filler wire.

(2) for the automatic argon arc welding machine and welding robot welding process with filler wire.

(3) for the laser welding process of automatic wire fill.

5.Cold filler wire machine (argon arc welding wire feeding machine) standard parts list

(1)wire feeding machine      1

(2)  wire filling gun          1

(3) wire feeding wheel       1

(4) wire feeding mouth     /0.8/1.0/1.2   each 1

6.The main function characteristic

(1) automatic wire filling speed can be adjusted continuously;

(2) drawing the wire back function;

(3) delay the wire feeding function;

(4) pulse feeding function;

(5)continuous wire feeding function;

(6) dual drive wheel feeding system, applicable to a variety of wire diameter of wire

(7) applicable to all kinds of argon arc welding machine;

(8) greatly enhance the work efficiency, reduce labor intensity, save labors and equipment cost.

(9) good welding effect is stable, improve the quality of products;

(10) various function parameters of independent continuous adjustable.

(11) adjusting the parameters with high accuracy.

(12) the standardization of products and components, according to the actual application of matching.

(13) can be widely used in aluminum alloy, stainless steel, welding of low carbon steel and non-ferrous metal and many other materials;

(14) argon arc welding, laser welding and other welding methods.

(15) and can be applied to artificial, the welding process automation and robotplane.

7.The main technical parameters：

Voltage：110V

The number of phases: phase

frequency:50/60Hz

Motor: DC permanent magnet

Speed:1200rpm

Wire table size:305mm

Wire size:0.6mm,0.8mm,1.0mm,1.2mm,

Wire feed speed range:0-800 cm/min

A pulsed wire feeding time: 0-10 Sec

Pulse pause time: 0-10 Sec

Time delay starting: 0-10 Sec

Back to silking time: 0-10 Sec

Power off time delay：0-5 Sec

Step control：4steps

External input voltage：DC  0~10V

Move the wire feeding speed：500 cm/min

Move back to the spinning speed：100 cm/min

High 420mm×wide 210 mm×length570 mm

Weight: 22 kg(48lbs)

Accuracy of speed: 1%；