SPE6045TW-3000W-110 Intelligent 3-Stage Temperature Adjustable Flash Dryer

- Installations

For convenient transportation, parts of the machine have been uninstalled. Operations

1. After well installed, connect the power supply.

2. Turn on the switch and then it will work automatically.

3. Put the probe of the thermometer on the drying table, heat for some time (20 minutes in reference), and then check the actual temp. If the temp is too low or too high, just risen or lower the height of the drying box.

- Reference Data

Based on our test, in the condition of 75℉ (24℃) room temperature and no air flowing around the dryer, when the flash dryer has been working for about 30 minutes, the maximum temp of 0.79” (2cm)) below the Drying Box can reach about 374℉ (190℃).

- Tips

The temp has keen relations with the distance from the drying box and continuous working time, in addition to the environmental factors (e.g. air flow, room temp, etc.).

Temp Test Instructions

As a equipment in screen printing, flash dryer play an important role. Based on the need of market and development of screen printing, this new flash dryer is designed specially. It is used to dry the screen. Compared with traditional dryer, it not only keeps the former advantages but also possess some new merits.

Features

- It adopts hierarchical (100 levels) temp-adjusting system, just click "+" and "-" to set hierarchical level for realizing accurate temp controlling.

(Note: The reading of the controlling panel doesn’t mean the temperature. The larger the number the panel displayed, the higher the temperature will be.)

- Equipped with six 500W far-infrared heating tubes and three switches. Each switch controls 2 tubes whose power is 1000W. Temperature can be adjusted in 3 stages. Easy operation and power—saving..

- High heating efficiency. Six stainless steel heating tubes (500W of each) make the temp increase quickly. The aluminum plate in the back of the tube can reflect heat, which means that the screen can be dried evenly.

- Foundation is paralleled with heating box. Stable, space-saving and safer.

- All chroming-steel structure. Sturdy and artistic design with powder coated housing to prevent rust.

- Height is adjustable. Equipped with four universal wheels, easy to move.

- Full set of installing tool equipped, easy to assemble.

Installation & Operation

For convenient transportation, parts of machine have been uninstalled. Please install them before using.

1. After well installed, connect power supply, turn on the switch and then it will work automatically.

2. Put the probe of the thermometer on the drying table, and then set the temp according to the needed drying temp and the provided referential data. Heated for six minutes, please check whether the temp it shows is same with the drying temp you need;

1) If the temp it shows is higher than your need, just click "-" to lower the digital reading appropriately;

2) If the temp it shows is lower than your demand, just click "+" to increase the digital reading appropriately.

Referential Data

(Notes: All the referential data we provide is the data showed in six minutes after each adjustment.)

Based on our test, with the normal air flowing,  the room temperature is 68℉ (20℃) and the distance between the drying box and the drying target is 1”(25cm).

- Stage 1 (with only one switch turned on)

1) If you set “3”, the actual temp of 1” (2.5cm) below the drying box is 89℉(32℃)

2) If you set “9”, the actual temp of 1” (2.5cm) below the drying box is 102℉(39℃)

3) If you set “12”, the actual temp of 1” (2.5cm) below the drying box is 129℉(54℃)

4) If you set “15”, the actual temp of 1” (2.5cm) below the drying box is 152℉(67℃)

5) If you set “18”, the actual temp of 1” (2.5cm) below the drying box is 183℉(84℃)

6) If you set “21”, the actual temp of 1” (2.5cm) below the drying box is 212℉(100℃)

7) If you set “24”, the actual temp of 1” (2.5cm) below the drying box is 258℉(126℃)

8) If you set “27”, the actual temp of 1” (2.5cm) below the drying box is 282℉(139℃)

9) If you set “30”, the actual temp of 1” (2.5cm) below the drying box is 293℉(145℃)

- Stage 2 (with two switches turned on)

1) If you set “3”, the actual temp of 1” (2.5cm) below the drying box is 98℉(37℃)

2) If you set “9”, the actual temp of 1” (2.5cm) below the drying box is 112℉(44℃)

3) If you set “12”, the actual temp of 1” (2.5cm) below the drying box is 141℉(61℃)

4) If you set “15”, the actual temp of 1” (2.5cm) below the drying box is 170℉(77℃)

5) If you set “18”, the actual temp of 1” (2.5cm) below the drying box is 204℉(96℃)

6) If you set “21”, the actual temp of 1” (2.5cm) below the drying box is 237℉(114℃)

7) If you set “24”, the actual temp of 1” (2.5cm) below the drying box is 291℉(144℃)

8) If you set “27”, the actual temp of 1” (2.5cm) below the drying box is 320℉(160℃)

9) If you set “30”, the actual temp of 1” (2.5cm) below the drying box is 330℉(166℃)

- Stage 3 (with all three switches turned on)

1) If you set “3”, the actual temp of 1” (2.5cm) below the drying box is 114℉(46℃)

2) If you set “9”, the actual temp of 1” (2.5cm) below the drying box is 131℉(55℃)

3) If you set “12”, the actual temp of 1” (2.5cm) below the drying box is 168℉(76℃)

4) If you set “15”, the actual temp of 1” (2.5cm) below the drying box is 204℉(96℃)

5) If you set “18”, the actual temp of 1” (2.5cm) below the drying box is 248℉(120℃)

6) If you set “21”, the actual temp of 1” (2.5cm) below the drying box is 289℉(143℃)

7) If you set “24”, the actual temp of 1” (2.5cm) below the drying box is 356℉(180℃)

8) If you set “27”, the actual temp of 1” (2.5cm) below the drying box is 390℉(199℃)

9) If you set “30”, the actual temp of 1” (2.5cm) below the drying box is 406℉(208℃)

Parameters

Input voltage: 110V

Power：1000w(Stage1), 2000W(Stage2), 3000W(Stage3)

Baking area: 18”x24” (45x60cm)

Net weight：

Range of Height between drying box and ground: 24”-40”（60-100cm）

ASC365 International Ltd

http://asc365.com